WOOD-ALUMINIUM DOOR DESIGN

batimet TA35 DSE

Formal Design and Construction Requirements

Wood-aluminium doors and wood-metal composite doors under the terms of this specification are designs whereby the outer side is covered by a metal profile. They have a material composition of wood and metal. The profiles arranged on the room side are laminates. The wood profiles take up the forces of horizontal traffic loads, wind loads, fill-ins, hardware etc. and distribute these forces accordingly. The metal profiles prevent direct weathering of the wood. Fixing of the aluminium frame to the wood frame is by means of rotatable and detachable clip holders in high quality temperature-resistant synthetic materials. Tension-free expansion of the diverse materials is guaranteed. The metal profiles are formed with a through-going ventilation and drainage system.

Wood Construction

The wood construction is carried out in conformity with the requirements of DIN 68121. Sashes and fixed glazing are carried out with screwed-on glass strips. Insulating glass edge seal is inserted into the wood rebate on both sides. Grooves in the wood profile for the insertion of the aluminium profiles are not permitted.

Aluminium Frames

Frame profile and sash profile form a flush surface plane. Between glass and sash profile there is an offset of 18 mm depth. If a sash overlapping panel forms the door’s sash, frame profile and aluminium panel equally are a flush surface plane. Drainage of the rebate area is by means of concealed punch holes in the bottom profile crosspiece of the frame. Optionally, there can be visible drainage with a cover cap.

The corner joints of the aluminium frames are welded.

Seal

The frame is added an all round, as frame functioning seal provided at the factory in high-quality TPE or EPDM between the wood frame and the wood sash. In the case of intensive driving rain, an additional seal can be mounted at the bottom between the aluminium frame and the aluminium sash.

Glazing on the outside is done with an all round APTK or EPDM glazing seal. Wedge seals are used as outer glazing seals for special fixed glazing. Internally, dry glazing with APTK or EPDM seals at graded levels of thickness are used. The system can be internally and externally installed as wet glazing.

Surface Coating for Aluminium

Coating is undertaken in conformity with valid quality guidelines for the coating of component parts in aluminium according to GSB-International GSB AL 631, or QUALICOAT regulations. The Principal can decide on the RAL or DB colour at his choice. Anodized surfaces are carried out mechanically to the corner joints of the aluminium frames.

Surface Coating for Wood

Surface treatments of wooden parts depends on the type of wood used, the selected coating system, and the expected wear & tear to the surface. If no dry layer thicknesses have been specified then the following minimum dry layer thicknesses required are in conformity with German rules:

> 30 µm for non-accessed surfaces and primed doors

> 50 µm in adjoining areas of building

> 80 µm for varnish coating

>100 µm for cover coating

This also applies to all surfaces under metal profiles and sheets which according to design, are not water drainage planes. All other surfaces require full layer thickness in the final treatment. The suitability of other coating systems and layer thicknesses on grounds of lower climate stress factors for wood-metal doors must be proven.

Demands on the Construction Design

The following properties and classifications are fulfilled in respect of the product norm for windows DIN EN 14351-1:

Profile system: batimet TA35 DSE

Laminated woods: (please select) fir/ pine/ oak/ hemlock/ Douglas fir

Glazing for doors and floor-connected glazing e.g. as set forth in GT 115

VSG 8 mm/ Argon SZR14/ Float 6 mm/ Argon SZR14/ VSG 8 mm

Panel filling in external aluminium sheet, thickness 3 mm, leaf overlap

DIN EN 12207 Air permeability Class 4

DIN EN 12208 Driving rain impermeability Class 9A

DIN EN 12210 Resistance to wind load Class 4

Leaf width: 600 – 1,600mm

Leaf height: 1,800 – 3,000mm

Leaf weight: to 300 kg as turning leaf only, 3-fold roller door band, batimet stainless steel

Opening to the inside with barrier-free threshold profile to DIN 18040-1

DIN 18008-4 fall protection TRAV Category A

To be provide by the principal

DIN EN ISO 10077-2 Heat insulation value: 	Uw _____ W/m²K

DIN EN ISO 140-3 Sound insulation: 		Rw _____ dB

DIN EN 1627 Break-in resistance: 		RC_____